| ○ ○ ○ ○ ● ○ Standard Symbol Definition and Purpose | Water conservation practices are activities that use water during the construction of a project in a manner that avoids causing erosion and/or the transport of pollutants off site. |
| --- | --- |
| Appropriate Applications | - Water conservation practices are implemented on all construction sites and wherever water is used. - Applies to all construction projects. |
| Limitations | - None identified. |
| Standards and Specifications | - Keep water equipment in good working condition. - Stabilize water truck filling area. - Repair water leaks promptly. - Vehicles and equipment washing on the construction site is discouraged. - Avoid using water to clean construction areas. Do not use water to clean pavement. Paved areas shall be swept and vacuumed. - Direct construction water runoff to areas where it can infiltrate into the ground. - Apply water for dust control in accordance with the Standard Specifications Section 10, and WE-1, “Wind Erosion Control.” - Report discharges to RE immediately. |
| Maintenance and Inspection | - Inspect water equipment at least weekly. - Repair water equipment as needed. |