| ○ ○ ○ ○ ● ○ Standard Symbol Definition and Purpose | Vehicle and equipment fueling procedures and practices are designed to minimize or eliminate the discharge of fuel spills and leaks into storm drain systems or to watercourses. |
| --- | --- |
| Appropriate Applications | These procedures are applied on all construction sites where vehicle and equipment fueling takes place. |
| Limitations | - Onsite vehicle and equipment fueling shall only be used where it's impractical to send vehicles and equipment off-site for fueling. |
| Standards and Specifications | - When fueling must occur onsite, the contractor shall select and designate an area to be used, subject to approval of the Resident Engineer (RE). - Absorbent spill clean-up materials and spill kits shall be available in fueling areas and on fueling trucks and shall be disposed of properly after use. - Drip pans or absorbent pads shall be used during vehicle and equipment fueling, unless the fueling is performed over an impermeable surface in a dedicated fueling area. - Dedicated fueling areas shall be protected from storm water run-on and runoff, and shall be located at least 15 m (50 ft) from downstream drainage facilities and watercourses. Fueling must be performed on level-grade areas. - Nozzles used in vehicle and equipment fueling shall be equipped with an automatic shut-off to control drips. Fueling operations shall not be left unattended. - Protect fueling areas with berms and/or dikes to prevent run-on, runoff, and to contain spills. - Use vapor recovery nozzles to help control drips as well as air pollution where required by Air Quality Management Districts (AQMD). Ensure the nozzle is secured upright when not in use. - Fuel tanks shall not be "topped-off." - Vehicles and equipment shall be inspected on each day of use for leaks. Leaks shall be repaired immediately or problem vehicles or equipment shall be removed from the project site. - Absorbent spill clean-up materials shall be available in fueling and maintenance areas and used on small spills instead of hosing down or burying techniques. The spent absorbent material shall be removed promptly and disposed of properly. - Federal, state, and local requirements shall be observed for any stationary above ground storage tanks. Refer to WM-1, “Material Delivery and Storage.” - Mobile fueling of construction equipment throughout the site shall be minimized. Whenever practical, equipment shall be transported to the designated fueling area. |
| Maintenance and Inspection | - Fueling areas and storage tanks shall be inspected regularly. - Keep an ample supply of spill cleanup material on the site. - Immediately cleanup spills and properly dispose of contaminated soil and cleanup materials. |