| ○ ○ ○ ○ ● ○ Standard Symbol Definition and Purpose | Procedures and practices to minimize or eliminate the discharge of pollutants to the storm drain systems or to watercourses from vehicle and equipment maintenance procedures. |
| --- | --- |
| Appropriate Applications | These procedures are applied on all construction projects where an onsite yard area is necessary for storage and maintenance of heavy equipment and vehicles. |
| Limitations | - None identified. |
| Standards and Specifications | - Drip pans or absorbent pads shall be used during vehicle and equipment maintenance work that involves fluids, unless the maintenance work is performed over an impermeable surface in a dedicated maintenance area. - All maintenance areas are required to have spill kits and/or use other spill protection devices. - Dedicated maintenance areas shall be protected from storm water run-on and runoff, and shall be located at least 15 m (50 ft) from downstream drainage facilities and watercourses. - Drip Pans or plastic sheeting shall be placed under all vehicles and equipment placed on docks, barges, or other structures over water bodies when the vehicle or equipment is planned to be idle for more than one hour. - Absorbent spill clean-up materials shall be available in maintenance areas and shall be disposed of properly after use. Substances used to coat asphalt transport trucks and asphalt-spreading equipment shall be non-toxic. - Use off-site maintenance facilities whenever practical. - For long-term projects, consider constructing roofs or using portable tents over maintenance areas. - Properly dispose of used oils, fluids, lubricants, and spill cleanup materials. - Do not dump fuels and lubricants onto the ground. - Do not place used oil in a dumpster or pour into a storm drain or watercourse. - Properly dispose or recycle used batteries. - Do not bury used tires. - Repair of fluid and oil leaks immediately. - Provide spill containment dikes or secondary containment around stored oil and chemical drums. |
| Maintenance and Inspection | - Maintain waste fluid containers in leak proof condition. - Vehicle and equipment maintenance areas shall be inspected regularly. - Vehicles and equipment shall be inspected on each day of use. Leaks shall be repaired immediately or the problem vehicle(s) or equipment shall be removed from the project site. - Inspect equipment for damaged hoses and leaky gaskets routinely. Repair or replace as needed. |