○

○

○

○

●

●

CC

CC

Standard Symbol

| Definition and Purpose | Concrete curing is used in the construction of structures such as bridges, retaining walls, and pump houses. Concrete curing includes the use of both chemical and water methods. Proper procedures minimize pollution of runoff during concrete curing. |
| --- | --- |
| Appropriate Applications | All concrete elements of a structure (e.g., footings, columns, abutments, stems, soffit, deck) are subject to curing requirements. |
| Limitations | - None identified. |
| Standards and Specifications | Chemical Curing - Avoid over-spray of curing compounds. - Minimize the drift of chemical cure as much as possible by applying the curing compound close to the concrete surface. Apply an amount of compound that covers the surface, but does not allow any runoff of the compound. - Use proper storage and handling techniques for concrete curing compounds. Refer to BMP WM-1, “Material Delivery and Storage.” - Protect drain inlets prior to the application of curing compounds. - Refer to WM-4, “Spill Prevention and Control.” Water Curing for Bridge Decks, Retaining Walls, and other Structures - Direct cure water away from inlets and watercourses to collection areas for removal as approved by the RE and in accordance with all applicable permits. - Collect cure water and transport or dispose of water in a non-erodible manner. See BMPs SS-9, “Earth Dikes/Drainage Swales & Lined Ditches,” SS-10, “Outlet Protection/Velocity Dissipation Devices,” and SS-11, “Slope Drains.” - Utilize wet blankets or a similar method that maintains moisture while minimizing the use and possible discharge of water. |
| Maintenance and Inspection | - Ensure that employees and subcontractors implement appropriate measures for storage, handling, and use of curing compounds. - Inspect any temporary diversion devices, lined channels, or swales for washouts, erosion, or debris. Replace lining and remove debris as necessary. - Inspect cure containers and spraying equipment for leaks. |