| ● ● ● ○ ○ ○ Standard Symbol Definition and SSV Purpose | Practices to remove tracked sediment to prevent the sediment from entering a storm drain or watercourse. |
| --- | --- |
| Appropriate Applications | These practices are implemented anywhere sediment is tracked from the project site onto public or private paved roads, typically at points of ingress/egress. |
| Limitations | - Sweeping and vacuuming may not be effective when soil is wet or muddy. |
| Standards and Specifications | - Kick brooms or sweeper attachments shall not be used. - Inspect potential sediment tracking locations daily. - Visible sediment tracking shall be swept and/or vacuumed daily. - If not mixed with debris or trash, consider incorporating the removed sediment back into the project. |
| Maintenance and Inspection | - Inspect ingress/egress access points daily and sweep tracked sediment as needed, or as required by the Resident Engineer (RE). - Be careful not to sweep up any unknown substance or any object that may be potentially hazardous. - Adjust brooms frequently; maximize efficiency of sweeping operations. - After sweeping is finished, properly dispose of sweeper wastes at an approved dumpsite in conformance with the provisions in Standard Specifications Section 7-1.13 . |