| ● ● ● ○ ○ ○ Standard Symbol Definition and Purpose | A tire wash is an area located at stabilized construction access points to remove sediment from tires and undercarriages, and to prevent sediment from being transported onto public roadways. |
| --- | --- |
| Appropriate Applications | - Tire washes may be used on construction sites where dirt and mud tracking onto public roads by construction vehicles may occur. - This BMP may be implemented on a project-by-project basis with other BMPs when determined necessary and feasible by the Resident Engineer (RE). |
| Limitations | - Requires a supply of wash water. - Requires a turnout or doublewide exit to avoid having entering vehicles drive through the wash area. |
| Standards and Specifications | - Incorporate with a stabilized construction entrance/exit. See BMP TC-1, “Stabilized Construction Entrance/Exit.” - Construct on level ground when possible, on a pad of coarse aggregate, greater than 75 mm (3 inches) and smaller than 150 mm (6 inches). A geotextile fabric shall be placed below the aggregate. - Wash rack shall be designed and constructed/manufactured for anticipated traffic loads. - Provide a drainage ditch that will convey the runoff from the wash area to a sediment trapping device. The drainage ditch shall be of sufficient grade, width, and depth to carry the wash runoff. - Require all employees, subcontractors, and others that leave the site with mud-caked tires and/or undercarriages to use the wash facility. - Implement BMP SC-7, “Street Sweeping and Vacuuming” as needed. - Use of constructed or prefabricated steel plate with ribs for entrance/exit access is allowed with written approval of RE. |
| Maintenance and Inspection | - Remove accumulated sediment in wash rack and/or sediment trap to maintain system performance. - Inspect routinely for damage and repair as needed. |