| Standard Symbol Definition and ● ● ○ ● ○ ○ Purpose | Wind erosion control consists of applying water and/or other dust palliatives as necessary to prevent or alleviate erosion by the forces of wind. Dust control shall be applied in accordance with Caltrans standard practices. Covering of small stockpiles or areas is an alternative to applying water or other dust palliatives. |
| --- | --- |
| Appropriate Applications | - This practice is implemented on all exposed soils subject to wind erosion. |
| Limitations | - Effectiveness depends on soil, temperature, humidity and wind velocity. |
| Standards and Specifications | - Water shall be applied by means of pressure-type distributors or pipelines equipped with a spray system or hoses and nozzles that will ensure even distribution. - All distribution equipment shall be equipped with a positive means of shutoff. - Unless water is applied by means of pipelines, at least one mobile unit shall be available at all times to apply water or dust palliative to the project. - If reclaimed water is used, the sources and discharge must meet California Department of Health Services water reclamation criteria and the Regional Water Quality Control Board requirements. Non-potable water shall not be conveyed in tanks or drain pipes that will be used to convey potable water and there shall be no connection between potable and non-potable supplies. Non-potable tanks, pipes and other conveyances shall be marked “NON-POTABLE WATER - DO NOT DRINK.” - Materials applied as temporary soil stabilizers and soil binders will also provide wind erosion control benefits. |
| Maintenance and Inspection | - Check areas that have been protected to ensure coverage. |