| ○ ○ ○ ○ ● ● Standard Symbol Definition and Purpose | Procedures and practices for the proper handling and storage of materials in a manner that minimizes or eliminates the discharge of these materials to the storm drain system or to watercourses. |
| --- | --- |
| Appropriate Applications | These procedures are implemented at all construction sites with delivery and storage of the following: - Hazardous chemicals such as: - Acids, - lime, - glues, - adhesives, - paints, - solvents, and - curing compounds. - Soil stabilizers and binders. - Fertilizers. - Detergents. - Plaster. - Petroleum products such as fuel, oil, and grease. - Asphalt and concrete components. - Pesticides and herbicides. - Other materials that may be detrimental if released to the environment. |
| Limitations | - Space limitation may preclude indoor storage. - Storage sheds must meet building & fire code requirements. |
| Standards and Specifications | General - Train employees and subcontractors on the proper material delivery and storage practices. - Temporary storage area shall be located away from vehicular traffic. - Material Safety Data Sheets (MSDS) shall be supplied to the Resident Engineer (RE) for all materials stored. Material Storage Areas and Practices - Liquids, petroleum products, and substances listed in 40 CFR Parts 110, 117, or 302 shall be stored in approved containers and drums and shall be placed in temporary containment facilities for storage. - Throughout the rainy season, each temporary containment facility shall have a permanent cover and side wind protection or be covered during non-working days and prior to and during rain events. - A temporary containment facility shall provide for a spill containment volume able to contain precipitation from a 24-hour, 25-year storm event, plus the greater of 10% of the aggregate volume of all containers or 100% of the capacity of the largest container within its boundary, whichever is greater. - A temporary containment facility shall be impervious to the materials stored therein for a minimum contact time of 72 hours. - A temporary containment facility shall be maintained free of accumulated rainwater and spills. In the event of spills or leaks, accumulated rainwater and spills shall be collected and placed into drums. These liquids shall be handled as a hazardous waste unless testing determines them to be non-hazardous. All collected liquids or non-hazardous liquids shall be sent to an approved disposal site. - Sufficient separation shall be provided between stored containers to allow for spill cleanup and emergency response access. - Incompatible materials, such as chlorine and ammonia, shall not be stored in the same temporary containment facility. - - - Materials shall be stored in their original containers and the original product labels shall be maintained in place in a legible condition. Damaged or otherwise illegible labels shall be replaced immediately. - Bagged and boxed materials shall be stored on pallets and shall not be allowed to accumulate on the ground. To provide protection from wind and rain, throughout the rainy season, bagged and boxed materials shall be covered during non-working days and prior to rain events. - Stockpiles shall be protected in accordance with BMP WM-3, “Stockpile Management.” - Minimize the material inventory stored on-site (e.g., only a few days supply). - Have proper storage instructions posted at all times in an open and conspicuous location. - Do not store hazardous chemicals, drums, or bagged materials directly on the ground. Place these items on a pallet and when possible, under cover in secondary containment. - Keep hazardous chemicals well labeled and in their original containers. - Keep ample supply of appropriate spill clean up material near storage areas. - Also see BMP WM-6, “Hazardous Waste Management”, for storing of hazardous materials. Material Delivery Practices - Keep an accurate, up-to-date inventory of material delivered and stored on-site. - Employees trained in emergency spill clean-up procedures shall be present when dangerous materials or liquid chemicals are unloaded. Spill Clean-up - Contain and clean up any spill immediately. - If significant residual materials remain on the ground after construction is complete, properly remove and dispose any hazardous materials or contaminated soil. - See BMP WM-4, “Spill Prevention and Control”, for spills of chemicals and/or hazardous materials. |
| Maintenance and Inspection | - Storage areas shall be kept clean, well organized, and equipped with ample clean-up supplies as appropriate for the materials being stored. - Perimeter controls, containment structures, covers, and liners shall be repaired or replaced as needed to maintain proper function. - Inspect storage areas before and after rainfall events, and at least weekly during other times. Collect and place into drums any spills or accumulated rainwater. |